JEA247ESAC

Units Replacement

General Informations

If when testing the system in “running” or during an acquisition, a red label appears indicating:

<<Unit xx Not Working>>

Unit xx may need to be substituted.

Before doing this, try to reset the system, powering it up again.

If the symptom persists, follow next instructions about unit substitution.

It may happens that the malfunction of one unit affects all the subsequent units.

For example, if you have a system based on 48 units, during “running” or in acquisition you could verify that units 40, 41, 42, 43, 44, 45, 46, 47,48 do not work.

(this case has been noticed very rarely)

This generally means that unit 40 prevents the correct functioning of the following units.

So you have to replace only unit 40.

Units Replacement

1 – Opening the box

To perform a Unit replacement you must have a screw driver equipped with the tool necessary to unscrew the 8 screws of each unit.

Then you need to have a programmer to set the number of the new unit.

Programmer:

Removing the 8 screws:

Unscrew the 8 screws and separate the two halves of the box. Pay attention to the wires connecting the electronic board to the blue halve.

Carefully detach the wire connecting the blue halve to the electronic board.

It is easy to remove the glue that keeps in position the connector.

It is not necessary to put it again.

If the unit to be replaced is equipped with trigger, the wires connecting the blue halve to the electronic board are two.

One of them is red and black and is that related to the geophone.

When reconnecting preserve the right polarity; the black wire must be external respect to the electronic board.

Remove the screw connecting the electronic board to the red halve.

Gently pull out the electronic board from its connectors experiencing a parallel force with fingers.

2 – Programming the new electronic board

To perform this operation you must connect the programming unit , with the (new) electronic board correctly inserted, to the PC where the software is installed.

Lunch the software and then select “code” command from the top menu.

This operation must be performed with a PC that has JEA247ESAC software installed.

Once the programmer is plugged, the boards blinks.

Remember that you absolutely must not execute this operation with JEA247ESAC system connected !

If you do this and program a number, you will alter the number of all the units !

Lunch the programm and set the port number related to the programmer; then execute the menu command “code”.

After executing “code“ command you will get:

Then press ok.

Write the password: dolang

Write the number you want and then read it again with READ command to verify that the number has been written.

3 – Closing the box

Reconnect the wires and close the box in this way:

Put the rubber ring (gasket) in its correct position.