Technical Data of JEA247ESAC

| Structure | PC connected by USB/RS485 to sampling units |
| --- | --- |
| Max sampling units number | 120 – for more than 24 a repeater is needed |
| Resolution | 24 bit |
| AD Converter | Sigma Delta |
| Sampling Frequency (Fs) | 250Hz, 500Hz, 1000Hz, 2000Hz, 4000Hz, 8000Hz, 16000Hz, 32000Hz |
| Number of samples | Fs > 500Hz, max Number 16384 |
| Number of samples | Fs = 500Hz/250Hz max number 5M samples if 24 channels, 2.5M if 48ch ….. , 1M if 120 ch. |
| Sampling duration | Fs = 500Hz more then 1h (24 ch) Fs = 250Hz more then 2h (24 ch) |
| Trigger | Each unit can be connected to a trigger wire of desired length, to work in opening or closing contact. |
| Power | Rechargeable battery unit included in USB/RS485 box – more than 7h autonomy with 24 units in continuous work. |
| Bandwidth | Up to 1 KHz, 2 poles low pass filter, DC component rejection |
| Input impedence | 1500 ohm |
| Nyquist frequency | 16000Hz at every Fs |
| Signal/RSM noise ratio | >124dB at Fs=1000Hz, input shorted |
| Connection | Units serially connected to PC by RS485 standard, at 1MHz baudrate |
| Software | Application for Windows XP, 7 and 8 allows easy setting of all acquisition parameters and produces a .sgy file for further analisys |
| Geophones | Each Unit (corresponding to a channel) can be connected to a geophone with desired specifications |